Flushing or Administering Medications Through a Central Line

Skill Competency Documentation

Student’s name: _______________________________________ Grade/Teacher:___________________

Person trained: ____________________________________ Position:_________________ Initials:_____

Person training: ____________________________________ Position:________________ Initials:_____

*THIS PROCEDURE IS TYPICALLY PERFORMED BY A REGISTERED NURSE

| Procedure Steps |  |  | Initial Demonstration |  |  |  |  |  |  |  |  |  |  | Return Demonstration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Date: |  |  |  |  |  |  | Date: |  |  |  | Date: |  | Date: |  | Date: |  |  |  |  | Date: |
| - Review health care provider’s order |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Ensure proper documentation of parent/guardian authorization to administer medication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Clean your workstation with soap and water or disinfectant wipe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Gather supplies and place on clean surface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Review the six rights of medication administration to be sure it is: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - The right student |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - The right medication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - The right dose |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Being given at the right time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Being given by the right route |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Being given for the right reason |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Ensure that the medication has not expired |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - If medication is expired do not administer it, contact parent/guardian and health care provider |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Review the student’s allergy status |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Explain the procedure to the student at their level of understanding |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Position student |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Wash hands |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Put on gloves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Place clean drape on workstation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - On clean workstation place the two saline syringes, medication syringe, and heparin syringe along with four 70% isopropyl alcohol pads or other antimicrobial swab as directed by student’s health care provider’s orders |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Prepare the saline and heparin syringes for administration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Remove the cap from the saline syringe being careful to not touch the inside of the cap or the tip of the syringe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Lay the cap down on your clean workstation with the inside of the cap facing up |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Remove air from syringe by pulling back on plunger, then pushing forward until all air is removed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Tap the syringe to move air bubbles, if needed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Pick up the cap, being careful not to touch the inside of the cap or the tip of the syringe and replace the cap on the tip of the syringe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Repeat step 14 for the other saline syringe and the heparin syringe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Inspect the medication syringe for air bubbles, leaks, cracks, particulate matter, and clarity of medication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Medication syringe should not need to have air removed if prepared by a pharmacy |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - If air bubbles are present, gently tap syringe and dispel air bubbles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Assist student in removing clothing to uncover the dressing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Examine tubing and cap to ensure that all components of the system are compatible and secured, to minimize leaks and breaks in the system |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Scrub the access cap for a time of no less than 15 seconds (or the amount of time indicated on student’s Individualized Health Care Plan (IHCP)), using 70% isopropyl alcohol (or cleaning agent as prescribed) and vigorous scrubbing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Allow access cap to air dry for at least 15 seconds, do not blow or fan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Hold on to the needleless syringe connector and attach the prefilled 10 ml normal saline syringe into the center of the cap while maintaining the sterility of the syringe tip |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - A smaller syringe may push too much pressure into the catheter and cause it to burst |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Unclamp the line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Slowly inject normal saline into vascular access device, noting any resistance or sluggishness of flow, and slowly aspirate until brisk blood return is obtained |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Inability to flush or absence of blood return from a central vascular access device requires further investigation about the causes, see below for Problems Flushing the Line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Push the normal saline into the catheter using the push-pause method (10 short boluses of 1 ml interrupted by brief pauses) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Clamp the line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Hold on to the needleless syringe connector and disconnect the syringe from injection port after flushing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - If medication is to be administered, review the six rights of medication administration again to ensure that it is for: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - The right student |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - The right medication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - The right dose |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Being given at the right time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Being given by the right route |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Being given for the right reason |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Scrub the access cap for a time of no less than 15 seconds (or the amount of time indicated on student’s Individualized Health Care Plan (IHCP), using 70% isopropyl alcohol (or cleaning agent as prescribed) and vigorous scrubbing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Allow access cap to air dry for at least 15 seconds, do not blow or fan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Hold on to the needleless syringe connector and attach the medication syringe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Unclamp the line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Slowly push the medication from the syringe per health care provider’s orders or over the recommended amount of time per drug manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Clamp line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Hold on to the needleless syringe connector and remove medication syringe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Scrub the access cap for a time of no less than 15 seconds (or the amount of time indicated on student’s Individualized Health Care Plan (IHCP), using 70% isopropyl alcohol (or cleaning agent as prescribed) and vigorous scrubbing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Allow access cap to air dry for at least 15 seconds, do not blow or fan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Hold on to the needleless syringe connector and attach the prefilled 10ml normal saline syringe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Unclamp the line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Flush with 10ml normal saline using the push-pause method |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Inject the normal saline flush at the same rate as the medication administration rate to ensure that the entire dose has reached the bloodstream |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Clamp line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Hold on to the needleless syringe connector and disconnect the syringe after flushing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Scrub the access cap for a time of no less than 15 seconds (or the amount of time indicated on student’s Individualized Health Care Plan (IHCP), using 70% isopropyl alcohol (or cleaning agent as prescribed) and vigorous scrubbing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Allow access cap to air dry for at least 15 seconds, do not blow or fan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Hold on to the needleless syringe connector and attach the prefilled Heparin syringe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Unclamp the line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Administer the Heparin flush to cap the lumen using the push-pull method |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Clamp the line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Hold on to the needleless syringe connector and remove the syringe from injection port after flushing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Discard used supplies in appropriate receptacles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Remove gloves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Wash hands |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Assist student in dressing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Document assessment, intervention and outcome in student’s health care record |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Follow up with student’s parents/guardian and health care provider as needed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Plan for monitoring flushing and medication administration via central venous line:

School Nurse Name: ______________________________ Phone Number: ___________________

Trainee’s signature:  _______________________________________________________________

School Nurse’s signature:  __________________________________________________________