Oxygen Saturation Monitoring using Pulse Oximeter

Skill Competency Documentation

Student’s name: _______________________________________ Grade/Teacher:___________________

Person trained: ____________________________________ Position:_________________ Initials:_____

Person training: ____________________________________ Position:________________ Initials:_____

| Procedure Steps | Initial Demonstration |  | Return Demonstration |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Date: | Date: | Date: | Date: | Date: | Date: |
| - Review health care provider’s order/Student’s IHCP |  |  |  |  |  |  |
| - Ensure proper documentation of parent/guardian authorization to perform this procedure |  |  |  |  |  |  |
| - Gather supplies and place on a clean surface |  |  |  |  |  |  |
| - Wash hands |  |  |  |  |  |  |
| - Explain procedure to student at a level they will understand |  |  |  |  |  |  |
| - Turn on pulse oximeter |  |  |  |  |  |  |
| - Select a distal extremity (usually a fingertip) that can be held still and is void of nail polish, false nail, moisture, and sweat |  |  |  |  |  |  |
| - Minimize excessive environmental light |  |  |  |  |  |  |
| - an accurate oxygen saturation reading requires that the pulse oximeter is able to consistently detect the student’s pulse |  |  |  |  |  |  |
| - all pulse oximeters have some form of light signal or bar graph that correlates with detecting the pulse; and a consistent high level of detection for at least 20–30 seconds is necessary to determine an accurate reading |  |  |  |  |  |  |
| - Place pulse oximeter probe on distal extremity |  |  |  |  |  |  |
| - Check operation of the equipment (audible beep, fluctuation of the bar of light or waveform on the face of the oximeter) |  |  |  |  |  |  |
| - Make sure that the light-emitting sensor and light-receiving sensor are aligned opposite each other |  |  |  |  |  |  |
| - Wait 20-30 seconds |  |  |  |  |  |  |
| - Read oxygen saturation level on pulse oximeter |  |  |  |  |  |  |
| - Remove pulse oximeter probe |  |  |  |  |  |  |
| - Turn off pulse oximeter |  |  |  |  |  |  |
| - Follow health care provider’s orders with appropriate follow up care |  |  |  |  |  |  |
| - Wash hands |  |  |  |  |  |  |
| - Document oxygen saturation level in student’s health care record |  |  |  |  |  |  |
| - Follow up with parents/guardian and health care provider, as necessary |  |  |  |  |  |  |
| - Clean pulse oximeter per manufacturer’s recommendations |  |  |  |  |  |  |
| Procedure for Continuous Pulse Oximetry: |  |  |  |  |  |  |
| - If the pulse oximetry is indicated to be continuous, the probe needs to be secured in place per manufacturer’s instructions |  |  |  |  |  |  |
| - An order for continuous pulse oximetry requires an oximeter with an alarm |  |  |  |  |  |  |
| - Alarm parameters are to be set per the student’s Individualized Healthcare Plan/Health care provider’s orders |  |  |  |  |  |  |
| - If the alarm sounds, the student requires immediate attention to determine if it is a “false alarm” (commonly due to excessive movement) or a true emergency that reflects a sudden deterioration in the student’s respiratory status |  |  |  |  |  |  |

School Nurse Name: ______________________________ Phone Number: ___________________

Trainee’s signature:  _______________________________________________________________

School Nurse’s signature:  __________________________________________________________