Daily Emergency Checks

| Equipment: | Check: |
| --- | --- |
| Emergency Go-Bag | Emergency supplies and equipment in Emergency Go-Bag: - Gloves - Portable oxygen (if ordered) with appropriate size Ambu-bag - Appropriate size Ambu-bag facemask (for emergencies when unable to reinsert a new tracheostomy tube) - Portable suction machine that can operate with battery or electricity - Battery should have a full charge - Sterile suction catheters - Sterile saline vials - Water-based lubricant - Distilled or sterile water - 4x4 gauze - Split gauze dressing (if student uses) - Unscented and dye free soap - Syringe, if needed for tracheostomy tube cuff - Sterile water for cuffed trach filled with water - Two spare tracheostomy tubes (including obturator) — one the size the student currently uses and one that is a size smaller in the event that the tube needs to be changed and there is difficulty passing it through the stoma - Obturator, if applicable - Spare tracheostomy ties - It is recommended to have spare tracheostomy ties already secured to spare tracheostomy - Blunt scissors - Emergency phone numbers - Pulse oximeter — may be optional if student is not on oxygen or mechanical ventilation |
| Portable suction | - Correct suction pressure is set - Proper catheters are available - Suction machine is plugged in (if applicable) or battery is fully charged - Any emergency suction equipment, i.e., DeLee suction trap is available |
| Resuscitation Bag | - Is with the student at all times - Intact and creates pressure when occluded - Correct Positive End-Expiratory Pressure (PEEP) valve setting - Auxillary ports closed - Pop-off valve is working - Oxygen tubing present and connects to oxygen supply |
| Oxygen Supply | - Properly connected to ventilatory - Connects properly to resuscitation bag - Oxygen tank is full - Wrench, connectors, and adapters are accessible |
| Emergency Oxygen Supply | - Properly connects to ventilator and resuscitation bag - Oxygen tank is full - Wrench, connectors, and adapters are accessible |
| Ventilator | - Primary and secondary settings are reviewed and correct - Test lung is available - Equipment is plugged in and charging - Backup set of tubing is assembled and accessible |
| Pulse Oximeter | - Proper probe placement - Good waveform - Review of prescribed alarm settings - Disconnect oximeter to ensure alarms are functioning and can be heard in the classroom setting - Equipment is plugged in, if applicable or battery is fully charged - Replacement probes are available |
| Medication | - Emergency medications in the correct doses are easily accessible (if applicable) in Emergency Go Bag - Expiration dates are current |
| Infusion therapy pump, if needed | - Adequate supplies - Review prescribed medications and doses - Power supply cable available - Battery is fully charged |
| Communication | - Phone(s) are working - Phone(s) are fully charged - Emergency phone numbers are documented and accessible |
| Food pump, if needed | - Adequate supplies of formula and feeding bags - Check power supply cable - Check battery charge status of pump |
| Monitoring devices | - Check that the following devices have a power supply and that the device is working properly (if applicable) - Blood pressure monitor - End-tidal CO2 monitor - Additional devices as needed |
| Nebulizer | - Compressor and nebulizer cup are working properly - New cup and medications are available - Oxygen tank is available to use during a power outage to run nebulizer |

Source: Estrem, et al., 2020